CO3 AT02

DESIGN TASK

Name : P.Karthik Kumar

Register Number : 192312416

ECA5614 – 5G/6G COMMUNICATION SYSTEM

### Aim :

To design a LEO satellite-assisted communication model that provides reliable wireless connectivity to remote and rural areas where terrestrial cellular networks are unavailable or difficult to deploy.

CODE :

clc; clear; close all;

% LEO Satellite-Assisted Remote Wireless Coverage

% Parameters

Re = 6371;                 % Earth radius (km)

h = 550;                   % LEO satellite altitude (km)

Rs = Re + h;               % Satellite orbit radius (km)

fc = 2e9;                  % Carrier frequency (Hz)

Pt = 30;                  % Transmit power (dBm)

Gt = 20;                  % Satellite antenna gain (dBi)

Gr = 5;                   % User antenna gain (dBi)

c = 3e8;                  % Speed of light (m/s)

% Ground coverage area

theta = linspace(0,2*pi,360);

coverage_radius = sqrt((Re+h)^2-Re^2);

% Satellite position

sat_x = 0;

sat_y = 0;

sat_z = Rs;

% Ground user grid

x = linspace(-1500,1500,100);

y = linspace(-1500,1500,100);

[X,Y] = meshgrid(x,y);

% Ground distance from satellite sub-point

D = sqrt(X.^2 + Y.^2);

% Maximum coverage condition

coverage = D <= coverage_radius;

% Slant distance approximation

slant_distance = sqrt(h^2 + D.^2);

% Convert distance to meters

d = slant_distance * 1000;

% Free-space path loss

FSPL = 20*log10(d) + 20*log10(fc) + 20*log10(4*pi/c);

% Received power

Pr = Pt + Gt + Gr - FSPL;

% Mask areas outside coverage

Pr(~coverage) = NaN;

% Display results

fprintf('LEO Satellite Altitude = %.0f km\n',h);

fprintf('Carrier Frequency = %.2f GHz\n',fc/1e9);

fprintf('Transmit Power = %.2f dBm\n',Pt);

fprintf('Maximum Approximate Coverage Radius = %.2f km\n',coverage_radius);

fprintf('Maximum Received Power = %.2f dBm\n',max(Pr(:)));

fprintf('Minimum Received Power = %.2f dBm\n',min(Pr(:),[],'omitnan'));

% Coverage plot

figure;

imagesc(x,y,coverage);

axis xy;

axis equal;

xlabel('Ground Distance X (km)');

ylabel('Ground Distance Y (km)');

title('LEO Satellite Remote Wireless Coverage');

colorbar;

colormap(jet);

% Received power plot

figure;

surf(X,Y,Pr,'EdgeColor','none');

xlabel('Ground Distance X (km)');

ylabel('Ground Distance Y (km)');

zlabel('Received Power (dBm)');

title('LEO Satellite Received Power Distribution');

colorbar;

view(45,35);

% Coverage boundary

figure;

plot(coverage_radius*cos(theta),coverage_radius*sin(theta),'LineWidth',2);

hold on;

plot(0,0,'o','MarkerSize',10,'LineWidth',2);

grid on;

axis equal;

xlabel('Ground Distance X (km)');

ylabel('Ground Distance Y (km)');

title('LEO Satellite Ground Coverage Boundary');

legend('Coverage Boundary','Satellite Sub-point');

OUTPUT :

2.

AIM :

To develop and simulate a task offloading strategy between edge and cloud servers in a 5G network by selecting the processing location based on communication and computation latency, thereby minimizing overall task execution delay.

CODE :

clc; clear; close all;

% 5G Edge-Cloud Task Offloading Strategy

N = 100;                         % Number of tasks

rng(1);                          % Reproducible results

% Task parameters

task_size = randi([10 100],1,N); % Task size (MB)

cycles = randi([1 10],1,N);      % Required computation (Giga cycles)

% Edge parameters

edge_speed = 5;                  % Edge processing speed (GCycles/s)

edge_delay = 5;                  % Edge communication delay (ms)

% Cloud parameters

cloud_speed = 20;                % Cloud processing speed (GCycles/s)

cloud_delay = 40;                % Cloud communication delay (ms)

% 5G transmission rate

data_rate = 1000;                % Mbps

% Transmission delay

trans_delay = (task_size * 8) ./ data_rate;

% Edge computation delay

edge_comp = (cycles ./ edge_speed) * 1000;

% Cloud computation delay

cloud_comp = (cycles ./ cloud_speed) * 1000;

% Total delay

edge_total = edge_delay + trans_delay + edge_comp;

cloud_total = cloud_delay + trans_delay + cloud_comp;

% Offloading decision

decision = strings(1,N);

for i = 1:N

if edge_total(i) <= cloud_total(i)

decision(i) = "EDGE";

else

decision(i) = "CLOUD";

end

end

% Count decisions

edge_tasks = sum(decision == "EDGE");

cloud_tasks = sum(decision == "CLOUD");

% Average latency

selected_delay = zeros(1,N);

for i = 1:N

if decision(i) == "EDGE"

selected_delay(i) = edge_total(i);

else

selected_delay(i) = cloud_total(i);

end

end

avg_delay = mean(selected_delay);

% Results

fprintf('Total Number of Tasks = %d\n',N);

fprintf('Tasks Processed at Edge = %d\n',edge_tasks);

fprintf('Tasks Offloaded to Cloud = %d\n',cloud_tasks);

fprintf('Edge Task Percentage = %.2f %%\n',(edge_tasks/N)*100);

fprintf('Cloud Task Percentage = %.2f %%\n',(cloud_tasks/N)*100);

fprintf('Average Selected Latency = %.2f ms\n',avg_delay);

% Display first 10 task decisions

fprintf('\nTask\tSize(MB)\tEdge(ms)\tCloud(ms)\tDecision\n');

for i = 1:10

fprintf('%d\t%d\t\t%.2f\t\t%.2f\t\t%s\n', ...

i,task_size(i),edge_total(i),cloud_total(i),decision(i));

end

% Plot 1: Edge vs Cloud latency

figure;

plot(1:N,edge_total,'LineWidth',1.5);

hold on;

plot(1:N,cloud_total,'LineWidth',1.5);

grid on;

xlabel('Task Number');

ylabel('Total Latency (ms)');

title('Edge vs Cloud Processing Latency');

legend('Edge','Cloud');

% Plot 2: Offloading decisions

figure;

bar([edge_tasks cloud_tasks]);

grid on;

xticklabels({'Edge','Cloud'});

ylabel('Number of Tasks');

title('Task Offloading Distribution');

% Plot 3: Selected latency

figure;

plot(1:N,selected_delay,'LineWidth',1.5);

grid on;

xlabel('Task Number');

ylabel('Selected Latency (ms)');

title('Latency After Task Offloading');

OUTPUT :